Constructing the phase diagram of sodium laurylethoxysulfate using dissipative particle dynamics

Maria Panoukidou a, Charlie Ray Wand a, Annalaura Del Regno b, Richard L. Anderson b, Paola Carbone a

a School of Chemical Engineering and Analytical Science, University of Manchester, Manchester M13 9PL, United Kingdom.

b STFC Hartree Centre, Scitech Daresbury, Warrington WA4 4AD, United Kingdom.

Corresponding authors: paola.carbone@manchester.ac.uk ; maria.panoukidou@manchester.ac.uk

(phone: +44 (0) 161 306 4367)

charlie.wand@manchester.ac.uk; annalaura.del-regno@basf.com; richard.anderson@stfc.ac.uk

## ABSTRACT

### Application

Sodium Laurylethoxysulfate (SLES) is a fundamental ingredient in a wide range of surfactant products and the mapping of its various mesophases is pivotal in predicting the liquid viscosity.

### Experiments

We use a novel set of Dissipative Particle Dynamics parameters specifically developed for surfactant molecules to construct the first phase diagram of pure SLES in sodium chloride/water solutions.

### Findings

We found that our DPD model is able to reproduce the range of morphologies expected for these types of ionic surfactants and in agreement with recent rheological data and theoretical predictions based on the packing parameter. We calculated the structure factor for various salt concentrations and show that the change from spherical to worm-like micelles can be inferred also looking at the intensity of the peak at intermediate q-values which decreases in intensity as salt concentrations increase. Varying the ethoxyl groups we observe that the additional ethoxyl group increased the micellar radius and affected the micelles’ shape polydispersity in the system. Finally, based on the contour length of worm-like micelles observed at intermediate salt concentrations, a closed mathematical formula is proposed capable of predicting the average micellar contour length given the salt and surfactant concentrations.

Keywords: Sodium Laurylethoxysulfate, anionic surfactants, DPD, contour length, packing parameter, structure factor.

## 1. INTRODUCTION

In aqueous solutions surfactants self-assemble into a collection of mesoscopic structures varying from discrete spherical and rod-like or worm-like micelles (WLM) to continuous branched networks [1]. This wide range of morphologies results in a variety of physicochemical behaviours that make surfactants the basic ingredients in the formulation of numerous commercial products used in multiple industrial sectors from the oil and gas [2] to the pharmaceutical industries [3,4]. Phase diagrams can be used to represent the various morphologies of surfactant systems. The phases depend on the surfactant chemical structure, its concentration (c), the presence of co-surfactants or other additives and, in the case of ionic surfactants, on the salt concentration (cs) of the solution. While the presence of co-surfactants has the main effect of reducing the interfacial tension, the presence of salt affects the solution morphology by screening the electrostatic repulsion between the charged surfactant head groups and allowing the formation of long micellar mesophases even at relatively low surfactant concentration [5].

In this work we have explored the phase behaviour of Sodium Laurylethoxysulfate (SLES), a common ingredient in several formulated home and personal care products. SLES is an anionic surfactant with the chemical formula CH3(CH2)m(OCH2CH2)nOSO3Na where n denotes the number of ethoxyl groups (EO) in the molecule (Fig.1) and m the number of carbons in the alkyl tail. The ionic strength of the solution and the number of ethoxyl monomers, n, strongly affect the micellar morphology [6,7]. Due to the divalent charged head group containing a sulfate functional group, the addition of salt to a SLES solution can lead to significant micelle growth and closed packed structures, while the number of EO groups have been shown to have a direct effect on the value of  critical micelle concentration (CMC) [8]. Commercial grades of SLES contain a variety of chemical impurities resulting from the synthesis methods adopted at large scale. For example they commonly contain inorganic salts (e.g., Na2SO4) resulting from the synthesis of the molecule. This synthetic by-product can accelerate the aggregation of the surfactants, enabling micelles to form at lower surfactant concentrations. The properties (specifically the viscosity) of the final mixture will be affected by the micellar size and the packing structure of surfactants [6,9,10].

Although SLES finds important applications in a wide range of sectors, published data on pure single component (i.e. with specific values of m and n) SLES systems are scarce. Indeed despite its apparently simple chemical composition, the synthesis of single component SLES is a costly and time consuming process [11]. Therefore the majority of published data refer to mixed systems of SLES in association with other surfactants [12–16] or industrially produced SLES which contains a distribution of alkyl chain lengths and EO groups [6].  So, while several computational and experimental studies, including the phase diagram, can be found on the related surfactant with n = 0 ethoxyl groups (i.e. sodium dodecylsulfate (SDS)) [17–19], we are unaware of any systematic studies of the phase behaviour and the structural properties of single component SLES in the literature.

The experimental characterization of a surfactant phase diagram is difficult and requires the use of several techniques including optical microscopy, fluorescence and scattering, however thanks to the recent increase in computer power and the introduction of coarse-grained molecular models, in-silico prediction of surfactant solutions properties is now possible. In particular, a mesoscopic modelling technique such as Dissipative Particle Dynamics (DPD) has proved to be a powerful tool to predict several surfactant solutions properties [20,21]. DPD has been extensively used to study surfactants self-assembly [22–24] and polymers solutions [25] to calculate CMC and investigate full phase diagrams. The rheology of surfactant solutions has been also investigated using non-equilibrium DPD simulations providing information on the orientation of WLMs obtained under shear rate before breaking apart [26–28].

The objective of this work is to construct a phase diagram for a system that consists of pure SLES of m = 11 (i.e. 12 carbons in the alkyl tail) and n = 1, SLES1EO. Moreover, we systematically investigate the effect that the number of EO monomers has on the self-assembly process and fully characterise the phase behaviour of the surfactant. Finally the mechanism of micellar elongation is investigated for SLES1EO and we construct a closed mathematical formula which is able to predict the average contour length of a micelle in the non-branching regions.

## 2. MATERIALS AND METHODS

2.1 DPD

Dissipative Particle Dynamics is a mesoscale simulation technique that has been used to explore many systems such as colloid suspensions [29,30] and polymers [31]. As it is a mesoscale method by nature, it deals with the movement of a cluster of atoms (beads) rather than individual atoms. It describes the movement of beads through Newton’s equations of motion and non-bonded interactions between the beads are most frequently characterized by soft repulsive potentials [32]. This method was initially introduced by Hoogerbrugge and Koelman in 1992 [33] while its mathematical background was established by Español and Warren in 1995 [34]. At the same time the ability of DPD to predict hydrodynamic behavior was proven by Español [35] by deriving two out of three conservative laws from DPD variables. This enables DPD to reach the thermodynamic equilibrium quicker than any equivalent coarse graining method. Also, as a mesoscopic method, DPD is able to reach time and length scales beyond those achievable by atomistic models and makes the simulations of any system faster than by simulating with molecular dynamics (MD).

For a full description of the DPD algorithm and theory the reader can consult the references above. Here we will just outline the basic concepts of the method. Let us consider a system of N particles that have equal mass. It is common in literature to perform this assumption for simplicity. A cluster of these N particles will form a DPD particle. The number of particles that constitute a bead is called the coarse graining parameter and is denoted by Nm [36].  Similarly to MD, the total force acting on a bead is the summation of several force components, in this case three distinct DPD forces and any other bonded force (eq. 2.1).

Fijtotal=FijC+FijD+FijR+Fijbonded			   	 (2.1)

where (FijC,FijD,FijR ) are the conservative, dissipative and random interaction forces between the beads. There are three coefficients in the interaction forces taking care of the lost degrees of freedom. The parameter aij is the conservative coefficient for beads i and j in FC, the dissipation coefficient γ in FD and rc is the cut-off radius beyond which no interaction between the beads is taking place. The random coefficient σ in the random force FR is related to γ through the following equation (2.2) while the random and dissipative weight functions are related to each other by equation (2.3).

σij2=2γijkBT				    	  (2.2)

wR(rij)=wDrij1/2 		    	          (2.3)

Relations (2.2)-(2.3) form the fluctuation dissipation theorem that enables DPD equations to establish an isotropic Galilean invariant thermostat which preserves hydrodynamics [34,37].

For the current work the dissipation parameter γ was taken equal to 4.5 for all the ij pairs of interactions and σ=3 whilst all the bonded interactions are described by a harmonic potential.

2.2 Simulation details

We adopt a model for the SLES1EO (n=1) in which each surfactant molecule consists of 9 beads in our DPD representation. In this representation one bead is a counter ion (Na+) that is not connected with bonded interactions to the rest of the surfactant. The model used for the coarse grained simulations is given in Fig.1 and the parameters used in DPD simulations are taken from Anderson et al. [38,39]. A table with all the interaction parameters can be found in the supporting document (Table S4). In similar manner SLES2EO and SLES3EO molecules consist of 10 beads and 11 beads respectively. The alkyl chain length in all the cases was 12 carbons which are translated into 6 hydrophobic tail beads (Fig.1).

Water is represented by beads consisting of two water molecules, Nm=2. Na+ and Cl- beads are conceptualised as being partially hydrated and modelled effectively as water beads but each possessing an additional charge [40]. This may seem quite a coarse approach for the charged beads but as yet no systematic approach exists in literature for dealing with charged anion and cation beads as previously discussed by Tang et al [16] and Anderson et al [39]. The salt concentration added in the system is given by cs so that the number of Na+ beads is the number of surfactants plus the number of added salt cations and the total beads of Cl- is the number of added salt anions. Since DPD is a coarse grained method the overlapping of beads will cause a thermodynamic instability if point charges are used [41,42]. For this reason charge smearing out of charges was accomplished by the exponential function given below [43]:

ρr=qπrλ2exp-2rλ    				     (2.4)

where λ is the decay charge length; here taken as λ=1.076 [38,39,43], q is the total charge and r the distance between two particles. In the current study, a significant proportion of the charge density is located outside of a given charged bead. In previous studies smaller λ values were trialled [40,44,45] and the effect of the shorter length on the calculated properties is negligible in this case.

All simulations were in NVT ensemble and were performed using DL_MESO mesoscale simulation package [46] in a cubic simulation box with volume of 64×103 DPD length3 (i.e. box dimension L=40 DPD length) and number density ρ=3. This box size has shown to be big enough to provide the correct CMC [39]. Throughout the study we explore multiple concentrations of surfactant. When 10% of the total number of particles were SLES1EO beads then c=0.100. More concentrations were also investigated, i.e. c=0.020, 0.050, 0.130, 0.150, 0.180, 0.200, so that a detailed phase diagram can be constructed.

The use of reduced units is common in DPD simulations. Correspondingly, the mass of a water bead is usually taken as the reference mass and the masses of the rest beads are reported with respect to the reference mass. In this model all the masses are approximated as equivalent to that of a water bead i.e. M = 1 DPD mass units for all particles. This is a common approximation in DPD simulations for computational efficiency [47]. The thermal energy was set to kBT = 1 DPD energy units with T = 298.15 K. In reference [38] the authors chose to separately specify the conservative coefficient aij and cut-offs rcij ranging from 0.950 to 1.170 DPD units. This reflects the chosen coarse-grained mapping and relative molar volume of the constituent atoms in each bead. The conversion from DPD to real length units is done by multiplying length by 5.65 Å for this model [38].

Fig.1: The Sodium Lauryl Ether Sulfate model adopted in this work. Each coloured circle signifies different DPD beads.  Blue for the alkyl chains, orange for the ethoxyl group, green is the sulphate bead and pink is the sodium cation.

Initially the surfactants and solution beads were randomly placed into the simulation box. Afterwards, each system was equilibrated using velocity Verlet integration scheme [37] with timestep dt = 0.020 DPD time units until the total number of micelles formed in the box converged to a constant value. There are two distinct self-assembly stages exhibited, the first stage is the rapid aggregation of surfactants into small clusters or micelles after a few hundreds of timesteps. Once the first stage is achieved the micelles undergo a second slower stage where the exchange of surfactants between them takes place. These two stages of self-assembly can be thought as equilibrium stages and the slowest one is taken into consideration when the thermodynamic equilibrium is checked. These two stages are visible in the plot of average number of micelles vs. time as in the first 104 DPD time units the total number of micelles decreases because smaller aggregates gather into larger micelles. It is also evident that the system reaches equilibrium more rapidly at low salt concentrations (see Fig.S1 in supporting document). Moreover, the total number of micelles in the system is smaller at higher salt concentrations indicating longer/larger aggregates. This happens as the increased salt in the system screens out the electrostatic interactions between the surfactants allowing them to pack closer.

Once the system has reached equilibrium, the production run is started and data collection commences for further analysis. The production runs last 4 ×105 DPD time units, i.e. 20 million timesteps for the longest simulation time and 105 DPD time units, i.e. 5 million timesteps for the shortest analysis. The accumulation frequency in the trajectory runs is the same in both cases and is every 100 DPD time units.

At the end of the production period a cluster analysis that counted the total number of micelles per frame and provided details on the total number of surfactants in each micelle along with the position coordinates of them, was performed over the whole trajectory. The cluster analysis was performed using UMMAP [48], using neighbour lists for efficiency. The systems were inspected visually by creating images using the Visual Molecular Dynamics (VMD) package [49]. A list of all the simulations performed can be found in supporting document (Tables S1 – S3).

2.3 Calculation of packing parameter

One of the properties that are calculated in the post processing is the packing parameter (pp). In the current work we will follow the geometry-based approach introduced by Israelachvili that describes the aggregation of surfactants into different micellar geometries [50]. This dimensionless number is associated with the principal curvatures of the micellar surface and the alkyl chain length of the surfactant.

It is known from theory [50] that when 0 < pp < 1/3 the geometry of the micelles is spherical, whilst if 1/3 < pp < 1/2 the micelles have rod or worm shape. At pp = 1/2 the shape is cylindrical and for values greater than 1/2 the observed structures have a vesicle shape, lamellar geometry or reverse micelle which are not observed in this work.

For the purposes of this work the pp is calculated in similar manner to ref. [51]. Initially the micellar surface was defined by the head group beads, i.e. CH2OSO3. Around each head group bead the neighbours within the distance Rc = 1.750 DPD length units were found. This led to a list of patches of beads on the micelle surface. In order to decide Rc the radial distribution function (g(r)) of the head groups was calculated with Rc taken as the first peak of g(r). Then the surface function given in equation (2.5) was fitted to the patches [51]

fx,y=Ax2+By2+Cxy+Dx+Ey+F 		   (2.5)

where the capital letters denote adjustable parameters and the coordinate of a point on the fitted patch is (x,y,fx,y). The packing parameters can then be found from the relation [50]:

pp = val=1-lH+l23K				(2.6)

where val is the notation used for the packing parameter and relates the volume of the head group, v, with the surface area a of head group attached on the micelle’s surface and the length l of the alkyl chain.

In equation (2.6) the parameter H refers to the mean curvature whilst K is the Gaussian curvature (see supporting document section s3). From the coefficients of equation (2.5) it is straight forward to calculate the curvatures of the surface:

K=4AB-C2					(2.7)

H=A+B 						(2.8)

Saddle points have a zero mean and negative Gaussian curvatures and for these points equations (2.7) and (2.8) do not give representative results of the micelle surface. In order to exclude the saddle points that could be fitted by the patches of particles described above, a constrained optimization was performed with the determinant of the optimization’s Hessian matrix to be strictly positive, i.e. the Hessian should be positive definite. In this way spherical or WLMs are more accurately fitted while branched aggregates that contain saddle points were excluded from the calculations. Finally, the time average of all pp is taken.

2.4 Structure factor

The structure factor describes how a material scatters incident radiation and can be calculated from simulations in two ways. The calculation of the structure factor can be achieved by the Fourier transformation of the radial distribution function:

Sq=1+4πρ0∞r2grsinqrqrdr       			  (2.9)

where ρ is the microscopic density and q the scattering vector. The minimum value of q in the simulations is set by the dimension of the simulation box and it is equal to  q=2πl, 0<l<L with l being in distance units. Alternatively the partial structure factor by bead type can be obtained by the Debye formula, i.e.

Sabq :=1i=1Nfi2j=1Nk=1Nfjfke-iq(Rj-Rk)    		    (2.10)

with f is the atomic form factor and R the positions of atoms (in our case the position of beads). In the present study the structure factor is calculated using the Debye formula (2.10) and the form factors are all assumed to be the same and equal to one [52–54].

2.5 Aspect ratio and contour length

The aspect ratio of a micelle is the ratio of its lengths in different orthogonal dimensions. The closer this fraction is to 1 the more spherical the shape is, thus for perfect spheres it is exactly 1.

In order to find the aspect ratio a micelle based axes coordinate system is defined. The centre of mass (COM) of each micelle is taken as the origin and the furthest point from the COM is found by calculating the greatest distance, rm, of a point on the surface from it. These two points define one axis and from this axis two more orthogonal axes are defined so that an orthonormal system of coordinates is formed. In case of spherical micelles the choice of x-axis is arbitrary and rm will give the radius of the micelle.

For spherical and rod-like micelles the aspect ratio will be meaningful, however in the case of WLMs the aspect ratio cannot be calculated as these structures are flexible and so do not span linearly in space. Instead, for these micelles the contour length will be calculated in similar manner to ref. [51]. Initially the micelles will be further coarse- grained by grouping the surfactants into larger superbeads. In order to do this a sphere of radius rsb is drawn around each head group with rsb taken to be the radius of a spherical micelle, rm. The number density ρ* of each sphere was calculated and the sphere with the highest ρ* was picked as a starting superbead. Around this initial superbead, n* new trial spheres are randomly generated at a distance of 2rsb from the initial superbead. Again ρ* was calculated for the trial sphere and the trial sphere with the highest value is selected, unless it is overlapping with a previously accepted superbead. This procedure is repeated until an end is reached, where the end is defined as having no trial spheres with ρ*>ρΤ, where ρΤ is threshold value. The accepted superbeads are then rechecked for trial spheres with ρ*>ρΤ to fully capture the surfactants in the micelle. By the end of this process the length of the superbeads gives the contour length of a micelle and this procedure was followed for all the micelles in the system.

## 3. RESULTS AND DISCUSSION

3.1 Phase diagram of SLES1EO

In this section the results for a concentration of c=0.100 SLES1EO with various salt concentrations are discussed. The addition of salt in the mixture is expected from literature to accelerate the aggregation of surfactants into micelles leading to longer aggregates. When the salt concentration reaches a certain value then branches can appear in the system [1]. The chosen DPD model was able to reproduce this behaviour and the most representative microstructures are shown in Fig.2. More specifically, the trend that is observed is an evident micelle growth with increasing salt concentration up to cs=0.100 where the first branches appear.

Fig.2: Final snapshot of c=0.100 SLES1EO with varying salt concentration at (a) cs=0.010 (b) cs=0.040 (c) cs=0.060 (d) cs=0.120 NaCl.

At cs=0.010 and cs=0.020 the micelles are spherical with the mean number of micelles decreasing from approximately 35 to 30 (Table 1). At cs=0.040, rods appear in the system while at cs=0.050 to 0.090 WLM, whose length increases with increasing salt concentration, develop. Some spherical micelles are observed for cs=0.050, 0.060 and 0.075 but their population reduces at cs=0.080 and cs=0.090. The full phase diagram is reported in Fig.3. The points (geometrical shapes) on the plot resulted from extensive simulations while the dotted coloured areas are based on a machine learning technique, the quadratic discriminant analysis algorithm (QDA). QDA is a classification method that separates the points by quadratic hyperplanes finding the optimal plane that fulfils two criteria: the distance between the centres of each class should be as close as possible and the variance within each class should be the minimum possible [55].

The concentration of surfactant is expected to shift the phase diagram towards the formation of larger aggregates even at relative small salt concentration. This is a result of the mass action law applied on micelles formation for which the excess amount of surfactants will increase the size of the existing micelles [56]. In accordance with the theory, Fig.4 shows that for surfactant concentration c=0.100 WLMs appear only when the salt concentration reached cs=0.050 while for c=0.130 SLES1EO these can be observed already at cs=0.040.

The distribution of micelles sizes shown in Fig.4 can give a better insight into the increased aggregation polydispersity observed with increasing salt concentration. Single surfactants, i.e. unimers, were excluded from the calculation of the aggregation number distributions. At high salt concentrations where only one structure was present in the simulation box as well as for the branched aggregates, no distribution of sizes was calculated. For the spherical micelles the distribution has a bell shape which however becomes broader and skewed towards longer aggregates as the salt level increases (Fig.4). The overlapping distributions at cs=0.050 and cs=0.060 is remarkable compared with the boost in micellar length at cs=0.075. This indicates that the extra amount of salt added to the system was not sufficient to favour further assembly of the micelles into longer structures and that this barrier is overcome only by reaching cs=0.075.

At low salt concentrations, cs=0.010, 0.020, the distribution of aggregation number behaves like a normal distribution and a mean and standard deviation (std) value can be extracted under this observation (Table 1). However as salt concentration increases, cs>0.040 the distribution consists of sparse peaks (Fig.4b). Several micellar shapes are observed to coexist in these cases and a mean value would not be representative of the system. For this reason we only report mean or std values for cs≤0.040.

Fig.3: Phase diagram of SLES1EO with salt concentration. The triangles (Δ) represent spherical micelles, circles (o) rod micelles, stars (*) WLM and diamonds (◊) branched micelles. The blue doted area represents the spherical micelles, green dots depict the rods, red is for WLM and black is for the branched structures.

Fig.4: Aggregation number of SLES1EO for c = 0.100 with (a) cs= 0.010, 0.020 and 0.040 (b) cs= 0.050, 0.060 and 0.075.

3.2 Contour length and packing parameter

In order to quantify the phase behaviour obtained from our simulations, the packing parameter was calculated following the methodology described in section 2.3. As observed in Fig.S2 (supporting document) the distribution of the packing parameter peaks at 0.225 for cs=0.010 and cs=0.020 and then the mean value increases with the salt concentrations reaching 0.494 at cs=0.040 and 0.564 at cs=0.050. All the mean values and stds are reported in Table 2. The results agree with the suggested theoretical values [50,57] within the confidence interval of the calculated error.

In the case of cs=0.050 the pp seems to be slightly higher than the theoretical predicted value of 0.500, although it is reasonable within the bounds of standard deviation. This is due to the quality of the fitting that was performed on the micellar surface as well as on the noise in the defined surface.

For spherical and rods micelles, complementary to the pp distributions is the calculation of micelle aspect ratio. For worm-like micelles where the calculation of aspect ratio would give less reasonable results, the contour length, ℓ, will be given. The probability distributions of aspect ratio from cs=0.010 to cs=0.040 salt concentration were found to behave as normal distributions with the mean value being 1 for cs=0.010, 0.020 and ~1.7 for cs=0.040. From the results, reported in Table 3, it is evident that micelles of spherical shape give aspect ratio close to unity while rods that have an ellipsoidal shape have a greater aspect ratio with the major axis being the nominator of the fraction. The aspect ratio between y and z axis in rods is close to unity indicating a circular shape at yz cross-section.

The probability distributions of ℓ for WLMs were calculated over a production run of 4×105 DPD time units and accumulation frequency in the trajectory file of 5 ×103 steps. As evident from Fig.5 not all the curves follow a normal distribution. Namely, for cs=0.050 the contour length distribution has a well-formed bell shape and for cs=0.060 it behaves like a binomial function while for cs=0.075 up to cs=0.090 two extreme peaks are observed at large lengths and fewer small micelles appear in the system. This leads to a shifting in the mean value that does not correspond to the most representative contour length found in the system (Table 4). The reason why the contour length distribution obtained for cs=0.060 is binomial (Fig.5a) is because in the system longer micelles of comparable size to those at  cs=0.075 coexist with shorter WLMs with sizes similar to those at cs=0.050.

The morphology for  cs=0.075 up to cs=0.090 is mainly dominated by one long micelle and several smaller micelles that coexist in smaller amounts (Fig.5b).

Fig.5: Contour length distribution function for worm-like micelles. In all of them c = 0.100 (a) cs=0.050 and cs=0.060 (b) cs=0.075, 0.080 and 0.090.

3.3 Structure factor

The structure factor (SF) was calculated using eq. (2.10) at c=0.100 for salt concentrations from cs=0.010 (spherical micelles) to cs=0.150 where branched aggregates had formed. To speed up the computation only the alkyl chain and the ethoxyl groups were taken into account. Inclusion of the head group in the calculations is not expected to change the qualitative behaviour of the curves but only to shift them slightly to the left. Water and salt beads were excluded from the calculations as we are predominantly interested in the underlying microstructure and the resulting SF did not show any specific structure due to the high solution beads to surfactant beads ratio.

The first point of the curves at the lowest q value corresponds to the simulation box size (Fig.6). For larger q the SF provides details of the microstructures formed in the system and in particular for q between 2π9.130=0.688 and 2π2.300=2.732 the first small shoulder peak is observed. This region represents the size of a spherical micelle (see Table 1 where micelle’s diameter is rm=2×4.565, thus q=2π9.130). The magnification of the curves in Fig.6 shows that the lower is the salt concentration the more intense is the peak. Indeed for cs=0.010 the spherical micelles dominate the system thus increasing the scattered intensity in this q-region; on the contrary at higher salt concentrations (above cs=0.050) spherical structures become the minority and thus the intensity is lowered. It is interesting to observe that the peak at this values of q never disappear completely even when the mesophases are rod- or worm- like as the radius of these larger “cylindrical” aggregates is still of comparable size of that of a spherical micelle (approximately twice the surfactant length). Values of q greater than 2.732 correspond to the size of one bead and all the curves overlap. The biggest difference between the curves is observed on the first point of the plot at low q due to the large structures that are formed at higher salt concentrations. For instance, at cs=0.150 only one big branched structure is present in the system. The system does not appear homogeneous to the incident radiation thus the scattered intensity is higher. Similar behaviour is observed in SDS solutions [58] and in SLES1EO/cocamidopropyl betaine (CAPB) mixtures [59] where the increasing salt concentration leads to a higher intensity value at low q values.

Fig.6: Structure factor of all the observed microstructures of SLES1EO. From blue cs=0.010 to red cs=0.150. Grey region is shaded because statistics are not accurate to these points.

3.4 Effect of different ethoxyl numbers

In order to compare the behaviour of SLES2EO and SLES3EO with SLES1EO, 4 more simulations were performed with salt concentrations at cs=0.020, 0.040 and c=0.130.

The presence of additional ethoxyl groups in the surfactant decreases the polydispersity of the aggregation number (Fig.7). For cs=0.040 the addition of one extra ethoxyl group leads to a reduced number of WMLs and eventually for SLES3EO no WLMs are formed (Fig.7b). Similarly for cs=0.020, increasing the number of EO groups reduces the likelihood to see rod-like micelles until the point where only spheres are observed for SLES3EO. Similar behaviour was observed experimentally in alkylpoly-oxyethyleneglycol molecules (CmEn) where the aggregation number was found to decrease as EO number was increasing [60]. Spheres still dominate in all the systems at cs=0.020 (Fig.7a) where the peak of the aggregation number lays at value commensurate with that of spherical micelles observed in section 3.1. For both salt concentrations (Fig.7a and Fig.7b) the histograms have the shape of a right- skewed distribution. In Table 5 mean and std are taken in the commensurate region of spherical micelles (i.e. first quantile of distribution).

Fig.7: Comparison of aggregation numbers of SLES1EO (red), SLES2EO (green) and SLES3EO (blue) for c = 0.130 and (a) cs = 0.020 (b) cs = 0.040.

Since the length of the surfactant is increased by one or two extra beads, the diameter of spherical micelles increases. For cs=0.020 we obtained 9.612(±0.166), 10.530(±0.124) and  12.315(±0.503) for SLES1EO, SLES2EO and SLES3EO respectively. It is interesting to notice that for SLES3EO the micelles had more an oblate shape rather than spherical one indicating a propensity to form structures with symmetry between rods and spheres. The aspect ratio was approximately 1.3 in both x/y and x/z directions. For cs=0.040 the contour length of the WLMs was found to be 24.348(±1.650) for SLES1EO. For SLES2EO the rod-like micelles had length 13.052(±0.296) on the longest direction; this decreased to 11.106±0.343 for SLES3EO.

3.5 Contour length predictive model for worm-like micelles

In this section we develop a predictive model for micelle elongation that does not include branched networks regions.  The contour length values of the micelles will be used as training points for an optimization scheme of a suggested nonlinear two-variable function. By observing the general behaviour of WLMs and measuring their contour length or the diameter of the micelles, 2rm, a polynomial surface can be fitted to the simulation data providing an estimate of the mean micelle length at concentrations of SLES/salt that are outside the simulated points. Since we are interested in the non-branched micelle regime, (cs,c) should take values from the region of spheres, rods or WLMs (see Fig.3 for phase behaviour). The surface that was fitted to the points is the following:

lcs,c=αc3+βcs3+γc2+δcs2+εcsc+ζc+ηcs+θ     	(3.1)

where Greek letters denote the fitting coefficients. The minimization problem that should be solved is given in eq. (3.2).

minχ∈R8lcs,c-l2 	    		          (3.2)

where χ=α,β,γ,δ,ε,ζ,η,θ is the set of the fitting parameters. The search method most efficiently suited for eq. (3.2) is the interior point trust region approach proposed by Coleman and Li [61]. According to this approach, the quadratic trust region sub-problem is approximately solved as the minimization of a quadratic problem subjected to an appropriate ellipsoidal constraint.

The training procedure was performed over 11 points randomly selected from the simulations performed and the optimal coefficients were found to be α= 470.845, β=139727.969, γ=314.024, δ= -29281.438, ε=2790.877, ζ= -59.967, η=1759.072 , θ= -21.437 with root mean square error 1.758. The resulting surface is given in Fig.8 and it is observed that for c = 0.100 the values reach a plateau between cs= 0.075 and cs= 0.090. This could be due to finite size effects. In order to keep the density of the system at ρ=3 the number of surfactants cannot exceed a maximum value and thus the length of the formed micelle should never exceed a specific length, in this case based on the box size, 24. The amplitude of the stem points in Fig.9 depicts the distance (error) of the training points from the fitted surface. The maximum residual observed was 3.175 and the minimum was -2.179.

The model was verified using a data set of 6 test points (Table 6) that lie within the optimization boundaries and the mean percentage error (MPE) is calculated to be 14.370%. In the obtained phase diagram the WLM region extends beyond the boundaries shown in Fig.8, i.e. at cs=0.120, c=0.050. However these regions are not included in the optimization process due to the significantly low number of micelles in the simulation box. Thus the regions that provide an insufficient sample were excluded from the domain of eq. (3.1).

With eq. (3.1) it is straightforward to calculate the expected contour length of the micelle without having to run extensive simulations by providing only the desired salt and SLES concentration within the given boundaries. This methodology can be also applied for other surfactant solutions and provides a relation between components’ concentrations with associated bulk properties, minimizing the number of simulations needed.

Fig.8: Optimal fitted surface on contour length points for SLES1EO. The dots represent simulation results; black dots are the training points and the red dots the testing points. The grey area depicts the fitted surface.

Fig.9: Residuals of training points from the optimal fitted surface in Fig.8.

## CONCLUSIONS

Due to the complicated and tedious synthetic route and the difficulty to experimentally characterise phase diagram of ionic surfactants [11,62], published data on the mesophases formed by these amphiphilic molecules are scarce and those available refer to the commercially available mixtures which comprise of surfactant with different aliphatic chain lengths and degree of ethoxilation [7,12–16]. Due to this wide range of impurities found in the samples, available experimental data such as CMC are therefore difficult to rationalize [63] making impossible to develop rigorous structure-property relationships. Since the rheological properties of the solution and the solubility of the surfactant depend on the morphologies of these mesophases, it is of fundamental importance to characterize pure component systems to disentangle the complicated interplay of enthalpic effects that lead to the formation of specific mesostructures.

In the present paper we present the first systematic study of the phase behaviour and the structural properties of single component SLES1EO surfactant focusing on the effects that the solution salinity have on the micellar morphology. The predicted phases (i.e. spherical, rods, worm-like micelles and branches) are in agreement with the few rheological measurements available [6] and with theoretical predictions based on the packing parameter [50].

In particular we observe spherical micelles at low salt concentrations (cs=0.010, 0.020) that transition to rods at cs=0.040. For salt concentrations from 0.050 to 0.090, worm-like micelles start to form which then exhibit branches for cs≥0.100.

The aggregation number was presented in probability distribution plots as a function of salt concentration and the packing parameters of these systems were calculated by fitting a second degree polynomial surface on groups of particles that define the micellar surface. The probability distribution of the packing parameter was reported showing a peak around 0.20 for spherical micelles and around 0.50 for rods and worm-like micelles confirming the morphologies assigned by visual inspection. The aspect ratio of spherical micelles was found to fluctuate around unity while in rods it resulted in a higher value on the micelle’s longest direction again confirming the presence of rods. The calculation of the structure factor confirmed the geometrical analysis and can be used in the future for direct comparison with experimental studies.

By comparing SLES1EO with SLES2EO, SLES3EO it was evident that the presence of longer ethoxyl chains within the surfactants resulted in a delayed appearance of rods and worm-like micelles. Surfactants with 3 ethoxyl groups form still spherical micelles at cs=0.040 indicating that the overall phase diagram for SLES2EO and SLES3EO is expected to be similar to that of SLES1EO but shifted to the right. It is interesting to notice that, counterintuitively, the presence of more ethoxyl groups tends to reduce polydispersity in terms of micellar shapes [60].

For worm-like micelles the contour length increases by increasing salt and SLES1EO concentration. Based on this observation a prediction model of micellar contour length that provides a closed mathematical relation was developed. This methodology can be used as a tool in the design of new surfactants systems to predict the contour length and associated bulk properties (e.g. viscosity), minimising the number of simulations and experiments required.

## ACKNOWLEDGEMENTS

This work was carried out as part of the EPSRC and Unilever funded ‘‘CAFE4DM’’ Prosperity Partnership grant as part of a research consortium involving the University of Manchester, the University of Cambridge, STFC and Unilever (EP/R00482X/1). R.L. Anderson and A. Del Regno were supported by the STFC Hartree Centre’s Innovation: Return on Research programme, funded by the Department for Business, Energy & Industrial Strategy. We would also like to show our gratitude to Dr. Patrick Warren (Unilever's R&D centre at Port Sunlight) for insightful discussions and proof reading the manuscript.

## REFERENCES

[1]	K. Holmberg, J. Bo, B. Kronberg, Surfactants and Polymers in Aqueous solution, John Wiley & Sons, Ltd., 2002. doi:10.1002/0470856424.

[2]	N.M.P. Rocha e Silva, H.M. Meira, F.C.G. Almeida, R. de C.F. Soares da Silva, D.G. Almeida, J.M. Luna, R.D. Rufino, V.A. Santos, L.A. Sarubbo, Natural Surfactants and Their Applications for Heavy Oil Removal in Industry, Sep. Purif. Rev. (2018) 1–15. doi:10.1080/15422119.2018.1474477.

[3]	P.A. Cornwell, A review of shampoo surfactant technology: consumer benefits, raw materials and recent developments, Int. J. Cosmet. Sci. 40 (2018) 16–30. doi:10.1111/ics.12439.

[4]	M.C. Prieto-Blanco, M. Fernández-Amado, P. López-Mahía, S. Muniategui-Lorenzo, D. Prada-Rodríguez, Surfactants in Cosmetics: Regulatory Aspects and analytical methods, in: Anal. Cosmet. Prod., Second Edi, Elsevier B.V., 2018: pp. 249–287. doi:10.1007/978-3-319-47164-8_6.

[5]	N.A. Mazer, G.B. Benedek, M.C. Carey, An investigation of the micellar phase of sodium dodecyl sulfate in aqueous sodium chloride solutions using quasielastic light scattering spectroscopy, J. Phys. Chem. 80 (1976) 1075–1085. doi:10.1021/j100551a011.

[6]	A. Parker, W. Fieber, Viscoelasticity of anionic wormlike micelles: effects of ionic strength and small hydrophobic molecules, Soft Matter. 9 (2013) 1203–1213. doi:10.1039/C2SM27078A.

[7]	H. Xu, J. Penfold, R.K. Thomas, J.T. Petkov, I. Tucker, J.P.R. Webster, The formation of surface multilayers at the air-water interface from sodium diethylene glycol monoalkyl ether sulfate/AlCl3 solutions: The role of the alkyl chain length, Langmuir. 29 (2013) 12744–12753. doi:10.1021/la4031848.

[8]	R. Sowada, On the Lipophilic Nature of Oxyalkyl Groups in Alkyl Ether Sulphates, J. f. Prakt. Chemie. 323 (1981) 269–278. doi:https://doi.org/10.1002/prac.19813230212.

[9]	T.F. Tadros, An introduction to surfactants, Cambridge University Press, Cambridge, 2014. doi:10.1017/CBO9781107415324.004.

[10]	Y. Moroi, Micelles, Theoretical and applied aspects, 1992. doi:10.1007/978-1-4899-0700-4.

[11]	A. Lavergne, Y. Zhu, A. Pizzino, V. Molinier, J.M. Aubry, Synthesis and foaming properties of new anionic surfactants based on a renewable building block: Sodium dodecyl isosorbide sulfates, J. Colloid Interface Sci. 360 (2011) 645–653. doi:10.1016/j.jcis.2011.04.110.

[12]	G. Petekidis, L.A. Galloway, S.U. Egelhaaf, M.E. Cates, W.C.K. Poon, Mixtures of colloids and wormlike micelles: Phase behavior and kinetics, Langmuir. 18 (2002) 4248–4257. doi:10.1021/la011751x.

[13]	M. Aoudia, T. Al-maamari, F. Al-salmi, Colloids and Surfaces A : Physicochemical and Engineering Aspects Intramolecular and intermolecular ion – dipole interactions in sodium lauryl ether sulfates ( SLES ) self-aggregation and mixed micellization with Triton X-100, 335 (2009) 55–61. doi:10.1016/j.colsurfa.2008.10.026.

[14]	M. Aoudia, B. Al-Haddabi, Z. Al-Harthi, A. Al-Rubkhi, Sodium lauryl ether sulfate micellization and water solubility enhancement towards naphthalene and pyrene: Effect of the degree of ethoxylation, J. Surfactants Deterg. 13 (2010) 103–111. doi:10.1007/s11743-009-1131-9.

[15]	H. Cheon, N. Jeong, H. Kim, Spontaneous Vesicle Formation in Aqueous Mixtures of Cationic Gemini Surfactant and Sodium Lauryl Ether Sulfate, 26 (2005) 107–114.

[16]	X. Tang, W. Zou, P.H. Koenig, S.D. McConaughy, M.R. Weaver, D.M. Eike, M.J. Schmidt, R.G. Larson, Multi-Scale Modeling of the Effects of Salt and Perfume Raw Materials on the Rheological Properties of Commercial Thread-like Micellar Solutions, J. Phys. Chem. B. 121 (2017) 2468–2485. doi:10.1021/acs.jpcb.7b00257.

[17]	A.M. Bellocq, J. Biais, B. Clin, A. Gelot, P. Lalanne, B. Lemanceau, Three-dimensional phase diagram of the brine-toluene-butanol-sodium dodecyl sulfate system, J. Colloid Interface Sci. 74 (1980) 311–321. doi:10.1016/0021-9797(80)90200-3.

[18]	P. Kékicheff, C. Grabielle-Madelmont, M. Ollivon, Phase diagram of sodium dodecyl sulfate-water system. 1. A calorimetric study, J. Colloid Interface Sci. 131 (1989) 112–132. doi:10.1016/0021-9797(89)90151-3.

[19]	Y. Shi-Ling, C. Zheng-Ting, X. Gui-Ying, Mesoscopic Simulation of Aggregates in Surfactant/Oil/Water Systems, Chinese J. Chem. 21 (2010) 112–116. doi:10.1002/cjoc.20030210205.

[20]	M.A. Johnston, W.C. Swope, K.E. Jordan, P.B. Warren, M.G. Noro, D.J. Bray, R.L. Anderson, Toward a Standard Protocol for Micelle Simulation, J. Phys. Chem. B. 120 (2016) 6337–6351. doi:10.1021/acs.jpcb.6b03075.

[21]	L. Rekvig, M. Kranenburg, J. Vreede, B. Hafskjold, B. Smit, Investigation of surfactant efficiency using dissipative particle dynamics, Langmuir. 19 (2003) 8195–8205. doi:10.1021/la0346346.

[22]	M.T. Lee, A. Vishnyakov, A. V. Neimark, Calculations of critical micelle concentration by dissipative particle dynamics simulations: The role of chain rigidity, J. Phys. Chem. B. 117 (2013) 10304–10310. doi:10.1021/jp4042028.

[23]	H. Wang, X. Tang, D.M. Eike, R.G. Larson, P.H. Koenig, Scission Free Energies for Wormlike Surfactant Micelles: Development of a Simulation Protocol, Application, and Validation for Personal Care Formulations, Langmuir. 34 (2018) 1564–1573. doi:10.1021/acs.langmuir.7b03552.

[24]	A. Vishnyakov, M.T. Lee, A. V. Neimark, Prediction of the critical micelle concentration of nonionic surfactants by dissipative particle dynamics simulations, J. Phys. Chem. Lett. 4 (2013) 797–802. doi:10.1021/jz400066k.

[25]	A. Prhashanna, S.A. Khan, S.B. Chen, Micelle morphology and chain conformation of triblock copolymers under shear: LA-DPD study, Colloids Surfaces A Physicochem. Eng. Asp. 506 (2016) 457–466. doi:10.1016/j.colsurfa.2016.07.003.

[26]	S. Meng, J. Zhang, Y. Wang, X. Li, C. Wu, T. Hou, L. Xiao, G. Lu, Simulating the rheology of surfactant solution using dissipative particle dynamics, Mol. Simul. 41 (2015) 772–778. doi:10.1080/08927022.2014.935373.

[27]	R. Pasquino, N. Grizzuti, P. Carbone, H. Droghetti, D. Marchisio, S. Mirzaagha, An experimental rheological phase diagram of a tri-block co-polymer in water validated against dissipative particle dynamics simulations, Soft Matter. 15 (2019) 1396–1404. doi:10.1039/c8sm01959b.

[28]	H. Droghetti, I. Pagonabarraga, P. Carbone, P. Asinari, D. Marchisio, Dissipative particle dynamics simulations of tri-block co-polymer and water: Phase diagram validation and microstructure identification, J. Chem. Phys. 149 (2018). doi:10.1063/1.5049641.

[29]	J.M.V.A. Koelman, P.J. Hoogerbrugge, Dynamic simulation of hard-sphere suspensions under steady shear, Eur. Lett. 21 (1993) 363. doi:10.1209/0295-5075/21/3/018.

[30]	P. Español, P.B. Warren, Perspective: Dissipative Particle Dynamics, 146 (2016) 1–18. http://arxiv.org/abs/1612.04574.

[31]	Z. Shen, H. Ye, M. Kröger, Y. Li, Aggregation of polyethylene glycol polymers suppresses receptor-mediated endocytosis of PEGylated liposomes, Nanoscale. 10 (2018) 4545–4560. doi:10.1039/c7nr09011k.

[32]	K.B. Lipkowitz, R. Larter, T.R. Cundari, Reviews in Computational Chemistry, John Wiley & Sons, Inc., Hoboken, NJ, USA, 2005. doi:10.1002/0471720895.

[33]	P.J. Hoogerbrugge, J.M.V.A. Koelman, Simulating Microscopic Hydrodynamic Phenomena with Dissipative Particle Dynamics, Europhys. Lett. 19 (1992) 155–160. doi:10.1209/0295-5075/19/3/001.

[34]	P. Espanol, P.B. Warren, Statistical mechanics of dissipative particle dynamics, Epl. 30 (1995) 191–196. http://iopscience.iop.org/0295-5075/30/4/001.

[35]	P. Espanol, Hydrodynamics from dissipative particle dynamics, Phys. Rev. E. 52 (1995) 1734–1742. doi:10.1103/PhysRevE.52.1734.

[36]	E. MOEENDARBARY, T.Y. NG, M. ZANGENEH, Dissipative Particle Dynamics: Introduction, Methodology and Complex Fluid Applications — a Review, Int. J. Appl. Mech. 01 (2009) 737–763. doi:10.1142/S1758825109000381.

[37]	R.D. Groot, P.B. Warren, Dissipative Particle Dynamics: Bridging the Gap between Atomistic and Mesoscopic Simulation, J. Chem. Phys. 107 (1997) 4423. doi:10.1063/1.474784.

[38]	R.L. Anderson, D.J. Bray, A.S. Ferrante, M.G. Noro, I.P. Stott, P.B. Warren, Dissipative particle dynamics: Systematic parametrization using water-octanol partition coefficients, J. Chem. Phys. 147 (2017) 094503. doi:10.1063/1.4992111.

[39]	R.L. Anderson, D.J. Bray, A. Del Regno, M.A. Seaton, A.S. Ferrante, P.B. Warren, Micelle Formation in Alkyl Sulfate Surfactants Using Dissipative Particle Dynamics, J. Chem. Theory Comput. 14 (2018) 2633–2643. doi:10.1021/acs.jctc.8b00075.

[40]	R. Mao, M.T. Lee, A. Vishnyakov, A. V. Neimark, Modeling Aggregation of Ionic Surfactants Using a Smeared Charge Approximation in Dissipative Particle Dynamics Simulations, J. Phys. Chem. B. 119 (2015) 11673–11683. doi:10.1021/acs.jpcb.5b05630.

[41]	M.E. Fisher, D. Ruelle, The Stability of Many‐Particle Systems, J. Math. Phys. 7 (1966) 260–270. doi:10.1063/1.1704928.

[42]	R.D. Groot, Electrostatic interactions in dissipative particle dynamics-simulation of polyelectrolytes and anionic surfactants, J. Chem. Phys. 118 (2003) 11265–11277. doi:10.1063/1.1574800.

[43]	M. González-Melchor, E. Mayoral, M.E. Velázquez, J. Alejandre, Electrostatic interactions in dissipative particle dynamics using the Ewald sums, J. Chem. Phys. 125 (2006) 224107. doi:10.1063/1.2400223.

[44]	K. Procházka, K. Šindelka, X. Wang, Z. Limpouchová, M. Lísal, Self-assembly and co-assembly of block polyelectrolytes in aqueous solutions. Dissipative particle dynamics with explicit electrostatics, Mol. Phys. 114 (2016) 3077–3092. doi:10.1080/00268976.2016.1225130.

[45]	P.B. Warren, A. Vlasov, Screening properties of four mesoscale smoothed charge models, with application to dissipative particle dynamics, J. Chem. Phys. 140 (2014) 084904. doi:10.1063/1.4866375.

[46]	M.A. Seaton, R.L. Anderson, S. Metz, W. Smith, DL_MESO: highly scalable mesoscale simulations, Mol. Simul. 39 (2013) 796–821. doi:10.1080/08927022.2013.772297.

[47]	R.D. Groot, K.L. Rabone, Mesoscopic simulation of cell membrane damage, morphology change and rupture by nonionic surfactants., Biophys. J. 81 (2001) 725–736. doi:10.1016/S0006-3495(01)75737-2.

[48]	D.J. Bray, A. Del Regno, R.L. Anderson, UMMAP- A statistical analysis software package for molecular modelling (in preparation), (2019).

[49]	W. Humphrey, A. Dalke, K. Schulten, VMD: Visual molecular dynamics, J. Mol. Graph. 14 (1996) 33–38. doi:10.1016/0263-7855(96)00018-5.

[50]	Jacob N. Israelachvili, Intermolecular and surface forces, Academic press LTD, 1991.

[51]	S. Dhakal, R. Sureshkumar, Topology, length scales, and energetics of surfactant micelles, J. Chem. Phys. 143 (2015) 024905. doi:10.1063/1.4926422.

[52]	B.L. Bhargava, R. Devane, M.L. Klein, S. Balasubramanian, Nanoscale organization in room temperature ionic liquids: A coarse grained molecular dynamics simulation study, Soft Matter. 3 (2007) 1395–1400. doi:10.1039/b710801j.

[53]	H.A. Karimi-Varzaneh, F. Müller-Plathe, S. Balasubramanian, P. Carbone, Studying long-time dynamics of imidazolium-based ionic liquids with a systematically coarse-grained model, Phys. Chem. Chem. Phys. 12 (2010) 4714–4724. doi:10.1039/b925780b.

[54]	A.R. Khokhlov, P.G. Khalatur, Microphase separation in diblock copolymers with amphiphilic block : Local chemical structure can dictate global morphology, 461 (2008) 58–63. doi:10.1016/j.cplett.2008.06.054.

[55]	G.A.F. Seber, Discriminant Analysis, in: Multivar. Obs., John Wiley & Sons, Inc., 1984: pp. 279–343.

[56]	Y. Moroi, Mass action model of micelle formation: Its application to sodium dodecyl sulfate solution, J. Colloid Interface Sci. 122 (1988) 308–314. doi:10.1016/0021-9797(88)90366-9.

[57]	K.D. Danov, P.A. Kralchevsky, S.D. Stoyanov, J.L. Cook, I.P. Stott, E.G. Pelan, Growth of wormlike micelles in nonionic surfactant solutions: Quantitative theory vs. experiment, Adv. Colloid Interface Sci. 256 (2018) 1–22. doi:10.1016/j.cis.2018.05.006.

[58]	M. Bergstrom, J.S. Pedersen, Structure of pure SDS and DTAB micelles in brine determined by small-angle neutron scattering ( SANS ), 1 (1999) 4437–4446.

[59]	W. Zou, G. Tan, H. Jiang, K. Vogtt, M. Weaver, P. Koenig, G. Beaucage, R.G. Larson, From well-entangled to partially-entangled wormlike micelles, Soft Matter. 15 (2019) 642–655. doi:10.1039/C8SM02223B.

[60]	R.. Zana, C. Weill, Effect of temperature on the aggregation behaviour of nonionic surfactants in aqueous solutions, J. Phys. Lettres. 46 (1985) 953–960. doi:10.1051/jphyslet:019850046020095300.

[61]	T.F. Coleman, Y. Li, An Interior Trust Region Approach for Nonlinear Minimization Subject to Bounds, SIAM J. Optim. 6 (1996) 418–445. doi:10.1137/0806023.

[62]	C. Bao, M. Ma, F. Meng, Q. Lin, L. Zhu, Efficient synthetic supramolecular channels and their light-deactivated ion transport in bilayer lipid membranes, New J. Chem. 39 (2015) 6297–6302. doi:10.1039/c5nj00937e.

[63]	W.C. Swope, M.A. Johnston, A.I. Duff, J.L. McDonagh, R.L. Anderson, G. Alva, A.T. Tek, A.P. Maschino, Challenge to reconcile experimental micellar properties of the cnem nonionic surfactant family, J. Phys. Chem. B. 123 (2019) 1696–1707. doi:10.1021/acs.jpcb.8b11568.

## TABLES

Table 1: Properties of SLES1EO for spherical and rod micelles. Mean is calculated as the standard average over time and std refers to standard deviation.

| Salt concentration and mean no of micelles | Mean aggregation no. (in surfactants) | std of probability distribution | Mean Radius of micelle (DPD length units) |
| --- | --- | --- | --- |
| cs=0.010 (35 micelles) | 51.848 | 21.465 | 4.565(±0.073) |
| cs=0.020 (30 micelles) | 68.201 | 27.538 | 4.694(±0.091) |
| cs=0.040 (20 micelles) | 102.667 (assuming normal distribution) | 60.656 (assuming normal distribution) | 6.790(±0.130) [longest direction] 4.427(±0.225) [y direction] 4.425(±0.234) [z direction] |

Table 2: Critical packing parameter (pp and its standard deviation, std) for SLES1EO.

| Salt concentration | mean pp | std of pp distribution |
| --- | --- | --- |
| 0.010 | 0.225 | 0.137 |
| 0.020 | 0.238 | 0.117 |
| 0.040 | 0.494 | 0.130 |
| 0.050 | 0.564 | 0.102 |

Table 3: Aspect ratios of c=0.1 SLES1EO for spherical and rod-like micelles. Standard deviation is indicated in brackets.

| Salt concentration | x/y | x/z | y/z |
| --- | --- | --- | --- |
| 0.010 | 0.999(±0.144) | 0.999(±0.147) | 1.000(±0.158) |
| 0.020 | 0.997(±0.150) | 0.999(±0.144) | 1.002(±0.147) |
| 0.040 | 1.689(±0.157) | 1.690(±0.155) | 1.000(±0.167) |

Table 4: Mean contour length and aggregation number of worm-like micelles for c=0.10 SLES1EO.

| Salt concentration | Mean ℓ in DPD length units | Std of ℓ | Most probable ℓ DPD length units |
| --- | --- | --- | --- |
| 0.050 | 20.195 | 3.105 | 20.270 |
| 0.060 | 23.265 | 3.811 | 22.165 |
| 0.075 | 21.999 | 4.540 | 24.592 |
| 0.080 | 21.521 | 4.262 | 23.852 |
| 0.090 | 20.927 | 5.776 | 23.837 |

Table 5: Mean aggregation number of different EO group numbers (in surfactants) at the region commensurate with spherical micelles at c=0.130.

| Number of EO | Mean aggregation no. at cs=0.02 salt |
| --- | --- |
| 1 | 72.251(±20.658) |
| 2 | 78.136(±29.439) |
| 3 | 87.662(±32.760) |

Table 6: Test points used for the verification of the predictive model of contour length for SLES1EO.

| [SLES1EO] | [NaCl] | Mean actual contour length | Mean predicted contour length |
| --- | --- | --- | --- |
| 0.100 | 0.040 | 13.580 | 19.796 |
| 0.100 | 0.060 | 22.160 | 23.235 |
| 0.100 | 0.080 | 23.850 | 23.369 |
| 0.120 | 0.050 | 19.970 | 25.663 |
| 0.130 | 0.040 | 24.350 | 24.076 |
| 0.150 | 0.050 | 32.660 | 31.370 |

Present address

BASF SE, Materials Molecular Modelling, Carl Bosch Str. 38, 67056, Ludwigshafen, Germany